# Description of site map

- There are 10 Central Distribution Racks (CD Rack) found on the site map with the main one containing the core switches/routers in CD Rack 1. The rest are running back to CD Rack 1 and are connected with a 100m duct containing 4km of single-mode fibre optic cabling. I have specified single mode as the distance of the cabling is more than 1km. Single-mode fibre offers much larger distance limits compared to multi-mode fibre.

- The fibre connections between the cabinets are ran in a ring with a redundant line through the middle. This shape is to allow redundancy between the racks if an area goes down. I also had the idea to go in a star-like pattern, where all lines are run back to CD rack 1, allowing for continuation of connectivity when one node goes offline. I eventually decided against this version as the ring method allows for better monitoring over the network.

- My reasoning for having so many CD racks is that by default CAT6 has a theoretical limit of 100 metres. By having 10 strategically placed CD racks on the site, it maintains that 100 metre limit and the connections receive full network access. There is an exception in CD rack 7 and CD rack 8. I have added two here for redundancy should one substation go offline. On top of this, not all visibility is gone should one go offline as the other rack is providing connectivity for the other cameras.

- Security Cameras have been placed at strategic locations throughout the site. With the current layout of cameras, no blind spots are present. They are all connected with 50mm ducts containing CAT6 data cabling. My reason for choosing CAT6 is because of the data speed of 10Gbps and they are low-cost compared to CAT6A. There was no requirement for fibre or faster network speed here as only video is being transmitted.

- The slow and fast chargers have been connected by 50mm ducts containing CAT6 network cables. These all then connect to a Central distribution Rack. My reasoning for networking the chargers are so that they can communicate with the network should an incident occurs where the chargers go offline, and even under a normal environment, to allow visibility and management of the chargers to users.

- I have also added outdoor access points in various locations outside. This would be for the following reasons:

  - Chargers – People waiting would have wifi access while they waited for their vehicle to charge fully.

  - Car Wash – Allows the functioning of a wireless dashboard to allow the user to maintain connectivity while their getting their car washed. On top of that, to add a quick note, the dashboard that is used to begin the wash would be wired using a CAT6 network cable to CD rack 9.

  - Battery Building – Should there be an issue with one of the buildings, a tech can receive connectivity without having to access the CD racks. It also allows for people passing by (for example from the car wash to the slow chargers) to remain connected to the network.